UNDERSTANDING SOLAR TERRESTRIAL REPORTS

PART II - INTERPRETING THE REPORTS

REVISION 1.2

ABSTRACT

Part I of this document discussed the morphology of

solar and geophysical phenomena.  With this background now

in hand, a discussion of the solar terrestrial reports

themselves can begin.  The purpose of this document is to

explain the meaning of the various sections of the Solar

Terrestrial Forecast and Review which are posted over the

networks on a weekly basis.  In addition, the purpose and

application of the other reports, alerts and warnings will

be discussed.  After having digested the material in parts

I and II of this document, the interested reader should

have enough background and knowledge to begin actively

applying the information in the reports.  The reader is

encouraged to digest part I of this document first (Part I

- Morphological Analysis of Phenomena).  It may be

obtained upon request from "oler@hg.uleth.ca".

July 14, 1991

UNDERSTANDING SOLAR TERRESTRIAL REPORTS

PART II - INTERPRETING THE REPORTS

REVISION 1.2

1.  Introduction

The solar terrestrial reports posted over the networks  presently

consist  of  several  reports, alerts and warnings.  The Solar Terres-

trial Forecast and Review is the only regular weekly publication.   It

contains  a  summary  of  conditions which occurred over the preceding

week and includes forecasts for the next 10 to 20 days.   This  report

is  the  one  which will be concentrated on most heavily in part II of

this document.  It contains most of the  data,  forecasts  and  charts

required and used in practical applications.

The Major Solar Flare Warning is a brief message which is  posted

over  the  nets when a major flare (or flares) may be possible.  These

messages are only sent when regions on the solar surface  are  complex

and   threatening   enough  to  produce  potentially  major  energetic

activity.  They are therefore only intended to  alert  people  to  the

increased potential for major flare activity.

A Major Flare Alert is posted whenever a  major  energetic  flare

erupts on the sun (of class M5.0 or greater).  Such an alert include a

description of the event and any  outstanding  accompanying  phenomena

(ex.  sweep frequency events, abnormally high radio bursts, etc.).  If

the flare could have a terrestrial impact,  an  impact  assessment  is

given within the body of the alert message.

The Major Geomagnetic Storm Alert is posted whenever  geomagnetic

conditions reach storm levels over middle latitudes.  These alerts are

not posted when storm conditions may exist for high latitudes, because

high  latitudes  experience a significantly greater number of magnetic

storms than do the lower latitudes and fewer  numbers  of  people  are

affected  by  the  high  latitude  storm  periods than middle latitude

storms.

We begin our discussion of the solar terrestrial reports with  an

analysis  of  the solar terrestrial review section of the reports.  We

will attempt to cover the language used and discuss the format of this

section  of the reports.  Following this, we will continue with a dis-

cussion of the Monthly Solar Terrestrial Review followed by a  discus-

sion  of  the  Geomagnetic Storm Alerts.  The Major Solar Flare alerts

and warnings should be more easily understood after this document  has

been digested.

The interested reader may need to re-read parts I and II of  this

document  before aquiring a clearer understanding of these reports and

their applications.  A great deal  of  material  is  covered  in  this

July 14, 1991

- 2 -

document  and  may  not  be  fully  understood the first time through.

Application of these reports to the various inter-related  fields  may

require practice and persistence in order to understand the impacts of

certain events on specific terrestrial systems (such as radio communi-

cations).  The interested reader is encouraged to do personal research

on the subjects of solar activity, ionospheric properties, radio  pro-

pagation  and geophysical activity.  Research in these areas will sig-

nificantly enhance ones understanding and  ability  to  interpret  and

apply  the information contained in the publicized reports, alerts and

warnings.

2.  The Solar Terrestrial Forecast and Review

This report is  the  primary  report  of  solar  and  geophysical

activity.   It  includes  enough  information and data to be of use to

many people involved in radio communications, solar physics  and  geo-

physics.   It  is  issued once a week and contains summaries and fore-

casts for the next 10 to 20 days.

The report itself is compiled from raw data obtained from several

sources.   One  of the major sources is the Space Environment Services

Center (SESC), which serves as a major global  data-collection  center

for space-related environmental data.  The SESC is responsible for the

solar terrestrial information which is posted on  radio  stations  WWV

and WWVH at 18 minutes past each hour.

The data obtained from the various sources are all collected  and

analyzed  by  before  being compiled into the reports which are publi-

cally posted.  Computer models, coronal maps  and  recurrent  patterns

are  all examined and analyzed.  The results are incorporated into the

various forecasts in the reports.  The actual prediction  methods  are

beyond the scope of this paper.

In this section, we will begin a systematic analysis of the vari-

ous  sections  of  the Solar Terrestrial Forecast and Review.  Some of

the terms contained herein may not  be  clearly  defined.   For  those

terms  which  are unclear, the interested reader is encouraged to con-

sult the "Glossary of Solar Terrestrial Terms", available upon request

from "oler@hg.uleth.ca".

2.1.  Summary of Solar Terrestrial Activity

This section of the report summarizes the highlights of solar and

geomagnetic activity which took place over the preceeding week.  Solar

activity is given first, followed by  a  summary  of  geomagnetic  and

auroral  activity.  Following this, a summary of the HF and VHF propa-

gation conditions for the preceeding week are given.  Any particularly

severe  solar  or terrestrial activity will be given special treatment

in this section.

Basically, the summary of solar activity includes a discussion of

those  regions  on the sun which exhibited abnormal signs of activity.

July 14, 1991

- 3 -

This may include a description of major solar flares, noteworthy fila-

ment  disappearances,  or  unusually large coronal holes.  It may also

include a description of various unusual or impressive forms of  limb-

activity  such  as  prominences,  plage  or  faculae activity, or limb

surges or flares.

In all of  the  solar  summaries,  references  will  be  made  to

specific  regions  postfixed  with specific numbers (ie. Region 6354).

These "region numbers"  are  simply  sequential  numbers  assigned  to

active  regions  as  they  appear or are identified.  The numbering of

these regions was started by the  SESC  many  years  ago.   The  first

region  to  be assigned was given a region number of 1.  Consecutively

identified regions were given numbers of 2, 3, 4 and so on.  Each  new

region is given the next consecutive region number.

In order for a region to be assigned a  region  number,  it  must

qualify  according to one of the following criteria: (1) If the region

has a sunspot group which has a first-postion  type-classification  of

C,  D,  E, F or H, it will be given a region number (see the "Glossary

of Solar Terrestrial Terms for a description of the sunspot  classifi-

cation  scheme).   (2)  If two or more reports confirm the presence of

class A or B spots (again, see the above-referenced document), it will

be  given  a region number.  (3) If the region produces a solar flare,

it will be given a region number.  (4) If the region  is  "bright"  in

H-alpha light and exceeds 5 heliographic degrees in either latitude or

longitude, it will be given a region number.  These four criteria  are

used  in  determining  what areas are assigned solar region numbers by

the SESC and which areas are not.

The vast majority of solar summaries include statements regarding

the  intensity  (or  class)  of  specific  flare  events.   Flares are

categorized using two types  of  classifications.   The  first  method

categorizes  a  flare  with  regards  to  its  output  energy at X-ray

wavelengths  measured  by  orbiting  satellites.   The  second  method

categorizes  flares  according to their size and brightness at optical

wavelengths (observed  using  monochromatic  H-alpha  light  filters).

Both  of these flare classifications are described fully in the "Glos-

sary of Solar Terrestrial Terms."  Refer to it for more information.

The positions of all solar regions and events are given according

to  the  format:  AxxByy, where xx represents a latitude (in degrees),

"A" represents either the "N" (North) or "S" (South) solar hemisphere,

"yy"  represents  the solar longitude given in degrees east or west of

the central solar meridian, and "B" represents either  "E"  (East)  or

"W" (West) of this central meridian (ex. N26E72).  The exact center of

the visible sun represents the origin where the longitude is  measured

from.   The extreme limbs of the solar disk represent longitudes of 90

degrees (either East or West, depending on which limb  you  look  at),

while  the  extreme  poles  of  the  sun represent 90 degrees latitude

(either North  or  South,  again  depending  on  which  hemisphere  is

observed).

It should be noted  that  the  orbit  of  the  earth  carries  us

slightly  above  and  below  the  suns rotational equator.  During six

July 14, 1991

- 4 -

months of the year, we are above the northern  portion  of  the  solar

equator,  while during the next six months, we fall below the southern

portion of the solar equator.  Near the  equinoctial  periods  (spring

and  fall), our orbit places us at our maximum distance above or below

the solar equator.  If, at these times of the  year,  the  earth  were

moved in a straight line toward the center of the sun, the earth would

make contact with the suns surface at a latitude of about 7.3  degrees

with  respect  to  the  solar  equator.  Although these periods do not

exactly coincide with the each equinox (ie. maximum  southerly  extent

is  achieved on 07 March, while maximum northerly extent is reached on

09 September), they do coincide within a month of the equinox.

These latitudinal changes are important, since it alters the  way

we  must  observe  the sun.  During the equinox periods, the center of

the sun as we see it is actually about 7 degrees to the north or south

of the actual solar equator.  If this were not taken into account, the

measured positions of sunspots and other  surface  features  would  be

grossly in error.

The important point to remember when studying  the  positions  of

sunspot groups is that the coordinates given represent the position of

the sunspots relative to the rotation axis of the sun as  viewed  from

earth.   For  example, a sunspot group located at a position of N21E62

represents a position 21 degrees north of the solar  equator,  and  62

degrees  east  of  the solar central meridian (or 28 degrees away from

the eastern limb [ 90 - 62 = 28 ]).

Since the sun rotates from east to west, all sunspot  groups  and

other  observed  features rotate in the same direction.  More specifi-

cally, the sunspots rotate at an average speed of about 13 degrees per

day.   So  the  sunspot  group  located  at N21E62 would be located at

N21E49 the following day, and N21E36 the day  after  that.   They  may

also occasionally drift in latitude, although the drift in latitude is

negligable most of the time.

Following the solar summary,  the  summary  for  geophysical  and

auroral  activity  is  presented.   These  summaries  should be mostly

self-explanatory with the exception of possible notes  regarding  mag-

netic fluctuations.

In summaries of particularly intense  magnetic  activity,  state-

ments  may be made regarding the maximum intensity of some of the mag-

netic fluctuations observed during the period being  reviewed.   These

summaries  will  generally  involve  the terms nanotesla and/or gamma,

which are synonymous.  The  intensity  of  magnetic  fluctuations  are

latitude-dependent.    Higher   latitudes  naturally  experience  more

intense magnetic fluctuations than  the  lower  latitudes.   Southerly

middle   latitude   regions  consider  magnetic  fluctuations  of  500

nanotesla (nT) to be very severe, while high  latitudes  may  consider

fluctuations  of  2500  nT  to  be  very severe.  The magnetic A and K

indices have been developed to aid in equating the intensity  of  mag-

netic fluctuations over wide latitudes.  For example, a magnetic fluc-

tuation at Anchorage Alaska  may  be  considered  to  be  as  "equally

intense"  as a similar fluctuation in California if the A or K-indices

July 14, 1991

- 5 -

for both locations are equal, even though the actual magnitude of  the

fluctuations at Anchorage are much higher than the corresponding fluc-

tuations in California.  A K-index of 4  at  Yellowknife  in  northern

Canada  may correspond to a magnetic fluctuation of 160 nT, while a K-

index value of 4 at Boulder Colorado may correspond to an actual  mag-

netic  fluctuation of only 50 nT.  Both fluctuations may be considered

equally severe based on how often fluctuations of that  magnitude  are

usually  encountered  for  that  latitude.   Yellowknife may encounter

fluctuations of 50 nT  on  a  daily  basis  whereas  Boulder  may  not

encounter  magnetic  fluctuations  of that magnitude for weeks.  Hence

the need for indices which can equalize the latitudinal dependencies.

Notes of auroral activity in the review section of the report are

generally  limited  to  descriptions  as  described in the Glossary of

Solar Terrestrial Terms.  However, for extraordinary  events  such  as

occur  during  auroral  storms, a more detailed examination of auroral

activity may be given.  Such descriptions may include  auroral  types,

color  fluctuations, pulsations or movement patterns of auroral forms.

All of these descriptions are  contained  in  the  Glossary  mentioned

above and part I of this document.

Notes regarding HF and VHF propagation are  usually  confined  to

brief  accounts  of  overall  global conditions.  These conditions are

generally rated as either above normal, normal, below normal  or  very

poor.   Above  normal  propagation  indicates strong signals which are

abnormally stable.  Above normal propagation is most often  associated

with good to very good DX potentials.  Normal propagation denotes nor-

mal conditions after considering the season and  the  position  within

the sunspot cycle.  It is compared with the average conditions experi-

enced over previous seasons and solar cycles.  Below  normal  propaga-

tion  is usually associated with increased geomagnetic activity and is

more consistent with signals of lower  quality,  less  stability,  and

weaker  strengths.   Chances  for  DX drop noticably during periods of

below normal propagation, except for the VHF bands where  an  increase

in DX may actually occur.  Very poor propagation is most often associ-

ated with magnetic storms or PCA events where signal absorption,  fad-

ing  and instability dramatically affect the quality of signals.  Dur-

ing intense storms, localized blackout  conditions  may  occur.   This

term  may be used in these instances to denote exceedingly high signal

absorption levels.  Again, the exception  is  VHF  frequencies,  where

long-distance  communications often improves during periods of high HF

absorption or blackout periods.  However, the quality of the VHF  sig-

nals may be quite poor despite the enhanced communication range.

2.2.  Short Term Solar Terrestrial Forecast

This section of the report follows the same  basic  structure  as

the  review described in the last section, except that predictions are

given instead of reviews.  The predictions are  made  using  the  same

methods  described  in the preceding sections, but are translated from

tables and charts into sentence form.

This short-term prediction section is intended to point  out  the

highlights which can be expected over the coming week.  Overall global

July 14, 1991

- 6 -

conditions are given in this section of the  report.   Therefore,  the

person  interested  in radio communications or auroral activity should

keep in mind the nature of this section.  It is not intended  to  list

the  possible localized phenomena which might occur.  Just the general

overall global conditions are stated.

The short term forecasts should be used as a guide only.  The art

of  predicting  geomagnetic  storms and major flares is by no means an

easy  process.   There  are  many  variables  which  are  unknown  and

processes  which  are not fully understood yet.  Although we have made

great advancements in the fields of solar physics and  geophysics,  we

have  a  long  way  to  go  in the area of predictions.  The forecasts

presented in these reports may therefore be in error at  times.   They

are,  however,  based  on  the most current models and the most recent

data.

2.3.  Solar Region Summary

The summary of solar regions is the section of the  report  which

is  in  tabular  form  and includes the region numbers, sunspot sizes,

sunspot classes, angular extents, magnetic configuration,  etc.   This

section  is of great value to those who are tracking sunspot groups or

watching for signs of growth or increased magnetic  complexity  and/or

flaring.

Although each of the aspects of this table are described  in  the

Glossary  of Solar Terrestrial Terms, we will elaborate on some of the

more vague terms of in this section.

Each solar region is given a number and its position on the solar

disk  is  measured  (as  was  described  above).   This identifies and

defines the exact position of a solar region on the  sun.   The  posi-

tional  description  (ie. the latitude/longitude description) is rela-

tive to the hemisphere of the sun which is  in  view.   That  is,  the

longitude  of  a solar region is relative to the center of rotation as

seen from the earth.  This places the 00  degree  longitude  (ie.  the

central  meridian)  continuously  at  the center of the sun (in a line

stretching from the north solar pole through the center of the disk as

observed  from  the earth, to the south solar pole).  All of the solar

regions rotate while the longitudinal lines remain  stationary.   This

method  of  marking  positions of sunspots and other phenomena is very

adequate, but fails to describe the position of sunspots  on  a  solar

global  basis  with  respect to a fixed 360 degree longitudinal system

(as is employed for the Earth).

In order to solve this problem, a system was developed  to  begin

mapping  active  regions  on  a  fixed  solar geographical basis.  The

actual longitudinal position of sunspots are therefore recorded in two

different  ways.   The first way (described in the preceding sections)

enables us to determine how far away a solar region is from  the  cen-

tral  meridian.   It  effectively  separates the observable solar disk

into an east and a west hemisphere with the dividing  line  coinciding

with  the  central  solar meridian.  The second method is analagous to

the way we have mapped  the  Earth,  with  fixed  lines  of  longitude

July 14, 1991

- 7 -

dividing up the entire sphere.

The  figures  in  the  region  summary  under  the  heading  "LO"

represent this second mapping method.  This second method is useful in

determining the movement of a sunspot region compared to the  flow  of

gases  around  the  sunspot  region.   Sometimes,  sunspots  will move

slightly slower than the gases around the spot, which  will  gradually

cause  the longitudinal location of the sunspot to change.  Sometimes,

they move faster than the gases normally do at that location.   So  by

observing  these longitudinal values, you can determine whether a sun-

spot is moving faster or slower than usual.

This method of referencing sunspots is also useful in identifying

regions  of  the sun which are abnormally active.  During the years of

maximum solar activity, the sun often  exhibits  longitudinal  regions

which  are  more  active  than other longitudes.  During solar maximum

years, there are often two areas of  abnormal  activity  separated  by

about  180 degrees.  By observing the positions of sunspots using this

method of mapping, the active  solar  longitudes  can  be  discovered.

This is valuable for those who want to forecast solar activity.  Like-

wise, some solar longitudes are often regions of enhanced  corpuscular

emissions  (ie.  regions  where matter is ejected from the sun), which

can  significantly  affect  radio   communications   and   geomagnetic

activity.   Plotting the positions of these active longitudes can also

be of tremendous aid in predicting  recurrent  storms  or  periods  of

increased geophysical activity.

The column in the table labelled "Z" represents an optical  clas-

sification  scheme  for  sunspots  and sunspot groups.  The details of

this classification method are given in the Glossary of Solar  Terres-

trial  Terms.  The interested reader is directed to consult this docu-

ment for more information.  It categorizes the optical shape and  com-

plexity of sunspot groups.

The column labelled "LL" represents the  angular  extent  of  the

sunspot  group.   Angular extent is given in solar degrees.  Comparing

this value with the number of spots within the region (denoted by  the

"NN"  column  of the table) yields the density of the group.  The den-

sity is important because it is an indirect measure of  the  gradients

of  magnetic  fields  within  the region.  High gradients produce more

frequent and more severe solar flares, while weak gradients  are  usu-

ally  associated  with  less-compact  spot  groups  which produce less

severe and less frequent flares.

The "MAG TYPE" or magnetic-type of sunspot groups as noted in the

last  column  of  the table can also be used to determine the magnetic

complexity and magnetic gradients within active regions.  Consult  the

Glossary mentioned above for more information regarding these classif-

ications.

In addition to details on spot groups, this region of the  report

also  enumerates  those  areas which are not associated with sunspots,

but contain areas of  enhanced  H-alpha  plages.   These  regions  are

assigned  region  numbers  according  to the rules noted above.  These

July 14, 1991

- 8 -

regions are often the sites for sunspot formation.  They may  also  be

associated with old regions which are decaying.

2.4.  Geomagnetic Activity Summary

Following the solar  region  summary,  a  graphical  analysis  of

recent geomagnetic activity is presented.  This graphical table charts

planetary geomagnetic activity as it is  recorded  for  many  magnetic

observatories  around  the world. It includes recent data for the last

96 hours up to the time the report was compiled.  The use of planetary

geomagnetic  activity  gives a good indication of global activity from

the high latitudes to the low latitudes.

This chart has been constructed from each of the 3-hourly K-index

values  reported  by  all of the participating magnetic observatories.

Each graph line, therefore, represents a 3-hour period of  time.   The

time  on  this  graph  is in Universal Time (relative from Greenwhich,

England).  Therefore, the first graph line of  this  chart  represents

the  activity  occurring  from 00 UT to 03 UT (actually, from 00 UT to

02:59:59 UT).  The second line represents activity occurring  from  03

UT to 06 UT, and so on.

The left-hand side of the chart relates the levels of geomagnetic

activity  to the approximate corresponding severity of activity.  This

activity is defined from "Very Quiet"  levels,  which  corresponds  to

magnetic  K-indices  of  zero,  to  "Extremely  Severe"  levels  which

corresponds to magnetic K-indices of about nine.

The right-hand side of this chart serves as a very rough guide to

the potential severity of magnetic-induction that might be experienced

during corresponding levels of magnetic activity.  By "magnetic induc-

tion,"  we  mean  the  severity  of magnetic fluctuations necessary to

begin influencing ground-based systems such  as  electrical  powerline

systems, telecommunications systems, pipeline networks, etc.  This end

of the chart is not intended to be a  definitive  classification,  but

rather  is  only  meant  to serve as a potential indicator to possible

magnetic-induction.  There are a great many  variables  that  must  be

taken  into  account before magnetic fluctuations can be qualitatively

classified as capable of inducing  electrical  currents  into  ground-

based  systems.   These  variables  are  not considered in this chart.

Only the general level of magnetic fluctuations are considered and are

related  to possible magnetic induction.  Such localized parameters as

air-earth conductivity, ionospheric current system parameters, electr-

ical  field configuration, ground resistivity, and ground-based system

network configurations must be considered (among other things)  before

true  hazards  regarding magnetic induction can be determined.  There-

fore, this area of the chart should be  used  only  as  a  very  rough

guide.   Nothing  more and nothing less.  It should be noted, however,

that magnetic fluctuations rated as K-indices greater than 6 generally

become  capable  of  wide-spread  electrical current induction. Storms

with  fluctuations  this  high  are  usually  capable  of  influencing

ground-based systems over wide areas.

The geomagnetic activity graphed in  this  chart  represents  the

July 14, 1991

- 9 -

peak  global magnetic activity observed during the respective periods.

It does not represent average magnetic activity.  This is important to

realize.   These  K-index  values  are not the same values reported on

radio stations WWV and WWVH.  The values reported  by  these  stations

represent the magnetic activity occurring at Boulder, Colorado.  Since

this chart is derived from measurements of geomagnetic activity around

the  world  (not  at  one specific location), the planetary values are

more valuable and applicable on a global scale.

2.5.  10-Day Geomagnetic Activity Forecast

This chart graphs the expected levels  of  planetary  geomagnetic

activity  over  a  10-day period.  Each day is separated into three 8-

hour segments.  Each line of the chart therefore represents one  eight

hour interval of time.  This chart graphs expected conditions relative

to Universal Time.  That is, the first line after each  date  dividing

line  represents  expected conditions between 00 UT and 08 UT for that

day.  The middle graph line represents conditions expected between  08

UT  and  16  UT.   The  last  graph  line  for  each  day in the chart

represents the magnetic activity that is expected from 16 UT to 24 UT.

This chart should be more easily interpreted than the previous geomag-

netic activity summary chart.  It is certainly more valuable.

The predictions are based primarily  on  data  regarding  coronal

holes,  potential  recurrent  activity,  diurnal  trends and potential

solar activity influences.  The transient solar component  (ex.  major

flares)  are not included as part of this prediction, since flaring is

extremely unpredictable and forecasts of potential  major  flaring  in

excess of a day or two is very unreliable.

2.6.  Graphical Analysis of Solar Activity

The graphical chart summarizing solar activity is  produced  each

week for a 60-day period.  This period covers two complete solar rota-

tions and is sufficient to show the cyclic behavior of solar  activity

from one cycle to another.

The solar flux (the intensity of solar radio  noise  observed  at

10.7  cm  wavelengths)  is  plotted  in this graphical analysis.   The

solar flux represents the slowly varying component  of  the  sun  (see

part  I)  and  is strongly correlated with the number and intensity of

sunspot groups on the solar surface.  The higher the  number  of  sun-

spots  visible,  the  higher  the  solar  flux.  As sunspots disappear

behind the western solar limb, the solar flux decreases.  The 10.7  cm

solar radio flux is therefore a good indicator of the overall state of

the observed solar environment.

Under normal conditions, the plot lines for the  solar  flux  are

plotted  using  asterisks  (*).   However,  on  days when major flares

erupt, these plot lines are changed from  asterisks  to  "F"'s.   This

enables readers to determine the period during the rotational cycle of

the sun when major  flares  occurred.   In  most  cases,  it  will  be

observed that most of the major flare activity occurs during the rota-

tional peak of each cycle.  There are, however, exceptions to this, as

July 14, 1991

- 10 -

will occasionally be noted.

Plot lines are only changed from  asterisks  to  F's  when  major

flares erupt which meet or exceed an X-ray intensity of class M5.0.  A

flare of class M4.9 may be a fairly major event, but is not considered

a  major  class  flare  since it never reached M5.0 class intensities.

Most flares, however, are either above or notably  below  this  limit.

There  are  many  more  flares  of class M3.0 intensity than there are

flares of class M4.0 intensity.  Most  major  flares,  therefore,  are

observed  to  occur  above  this  M5.0 transition level.  Very few are

borderline cases.

2.7.  20-Day Solar Activity Forecast

The 20-day solar activity forecast chart is constructed based  on

the  activity  which  was  observed  over previous solar rotations, in

addition to the status of the regions which are currently  visible  on

the  sun.   The  intensity, size and number of sunspots in each region

are all analyzed (among other things) before this prediction chart  is

produced.

The plot lines of this chart  represent  the  solar  flux  levels

which  are  expected  to occur throughout the 20-day period covered by

the chart.  The actual flux values will  frequently  differ  from  the

actual  flux  values  observed, since predicting the activity of solar

regions is still very difficult to do beyond approximately  one  week.

Regions  behind  the  sun  may be developing which could significantly

alter the shape of the prediction charts.   These  regions  cannot  be

seen  or  detected  in any way until they approach the eastern limb of

the sun.  Hence, these solar activity predictions should be used  only

as  a  guide.   The  predictions are generally good at forecasting the

times when the solar flux will peak or  reach  its  minimum  during  a

rotational  cycle,  and  this  can  be  of  tremendous value to people

interested in the level of ionospheric  ionization  which  is  propor-

tional to the solar flux.

Flares are not included in this  prediction  of  solar  activity.

Flares  are  extremely  difficult  to predict, even in the short-term.

Our knowledge of flares has grown rapidly since the early part of this

century.   However,  our knowledge is still not sufficient to reliably

predict the occurrence of major  flares  over  periods  in  excess  of

several  days.  Therefore,  this  graphical solar activity forecast is

limited to a treatment of the solar flux only.  As far as  flares  go,

an  increasing  solar  flux  generally  increases  the  risk for major

flares.  The higher the solar flux values, the greater  the  risk  for

major  flares,  since the solar flux is directly related to the number

and intensity of active regions on the sun.

2.8.  HF Radio Signal Propagation Predictions

This  section  of  the  Solar  Terrestrial  Forecast  and  Review

involves  the  propagation  of  high-frequency  (HF)  radio waves over

long-distances.  It is a forecast of the expected quality of HF  radio

signals travelling over long-distances.

July 14, 1991

- 11 -

The quality of radio  signals  is  divided  into  several  areas.

Radio  signals  which  have  outstanding  strength  and stability over

long-distances are categorized as  excellent.   These  conditions  are

rarely  observed  and  occur  more frequently over the lower latitudes

than the high latitudes.  Signals  which  are  abnormally  strong  and

stable  over  long-distances  are  classified as very good.  These are

above-normal conditions when considering the  time  of  year  and  the

state  of  the  solar cycle.  Signals which are normal for the current

season and state of the solar cycle are given a  good  classification.

These  signals  are generally stable and relatively strong considering

the time of year, but may suffer  some  minor  fading  or  distortion.

Noise  may  also  be somewhat of a factor, but is generally tolerable.

When signals fall below the normal quality, they may be categorized as

poor.   Poor radio signals over long distances are those which experi-

ence moderate to strong fading or flutter, abnormally high  levels  of

absorption,  or  increased  levels of noise (or any combination of the

above).  Long-distance propagation is still usually possible in  these

cases,  but  suffer significantly increased levels of distortion which

may hamper attempts at long-distance voice contacts.  Very poor  radio

signal  propagation  occurs  when signals experience severe fading and

flutter, high levels of absorption, high levels of noise and high lev-

els  of  distortion  (or any combination of the above).  Long-distance

communication usually becomes very difficult during these periods  and

may  not be possible at all over some regions.  When radio signals are

unable to be propagated at all over long distances, or are  very  poor

over  short  to  moderate  distances,  communication is rated as being

extremely poor.  This category will only  usually  be  encountered  at

higher latitudes and during periods of intense geomagnetic storming.

In these forecasts, each day is composed of three  8-hour  inter-

vals.   These  forecasts  are  also correlated with local time, not UT

time as are the geomagnetic forecasts.  The first plot  line  of  each

day  represents  the  interval  between 00 (midnight) and 08 am, local

time.  The second plot line represents the period between 08 am and 04

pm (or 08:00 to 16:00) local time and so on.

There is one very important note which should  be  understood  by

all  those  who  use  these  forecasts  as  guides.  The local time of

attempted communications is a very important factor  in  long-distance

communications  due  to the diurnal component.  This diurnal component

is not considered in these forecasts, nor could it  be  easily  incor-

porated  into  these  charts.   The charts are intended to be globally

valid. Hence, the obvious diurnal enhancements which occur in  differ-

ing ways for different regions cannot be included in this global fore-

cast.  The  person  interested  in  radio  communications  is  already

expected  to  have  a knowledge of the diurnal enhancements for his or

her region.  These charts, therefore, are only  intended  to  aid  the

interested  communications operator in determining the potential times

when enhanced  radio  communications  may  be  possible.   It  is  not

intended  to  reflect the diurnal enhancements which occur, unless the

enhancements are significant.

The forecasts are based  heavily  on  recurrent  geomagnetic  and

auroral activity, which are primary factors in determining the quality

July 14, 1991

- 12 -

of radio signal propagation conditions.  The intensity  of  ionization

of the appropriate ionospheric layers are also examined when preparing

these charts.

This section is separated into three charts for the high latitude

regions,  middle  latitudes  and  the  low  latitude  regions.  Global

separation of areas into  latitudinal  zones  is  required  since  the

characteristics  and  quality of radio propagation differ from zone to

zone.

To make the best use of these charts, the  interested  reader  is

encouraged  to  follow this procedure.  Determine the path endpoint of

your signal.  That is, determine the  location  where  you  want  your

transmitted  signal to be received.  This is the path endpoint or des-

tination.  Your transmitter location is the startpoint or source.  Now

draw  a  great-circle  between the startpoint and the endpoint.  Next,

determine the most northerly geographical coordinates  of  the  great-

circle  connecting  the startpoint and the endpoint (we will call this

point the northpoint) and note the current local time  at  the  north-

point.  After calculating this information, determine what latitudinal

zone the path northpoint lies in. Finally, consult the HF  propagation

prediction   charts   and  select  the  latitudinal  zone  chart  that

corresponds to the latitude of the path northpoint.  Using  the  local

time  at  the path northpoint, select the appropriate day in the chart

and examine the plot line which corresponds to that local time.   This

is  the propagation quality that can be expected for that path at that

time. Note, however, that you must also  consider  the  local  diurnal

signal  behavior  of  your  transmission, and the local diurnal signal

behavior at the path  endpoint  in  order  to  determine  the  diurnal

characteristics  that  should  be  expected.   This information is not

given in these charts, but should already be known by the radio opera-

tor  who is familiar with the diurnal conditions which occur at his or

her site.  In order to be most accurate, this diurnal  component  must

be considered together with the propagation predictions. Therefore, if

your transmission were conducted during a period of time when you know

both  the  startpoint  and  endpoint  signals  are  enhanced,  a truer

representation of the propagation quality may be obtained by examining

the  prediction  charts  (using  the  method above) and increasing the

quality of propagation up by no more than one level (ie.  from  "fair"

to "good").

For example, suppose you wanted to  communicate  between  Florida

and  Great  Britain.  Florida is a low latitude zone and Great Britain

is a middle-latitude zone.  Next,  we  draw  a  great  circle  between

Florida  and  Great Britain.  If you have no numerical method of doing

this, you can approximate the great circle path by stretching a narrow

piece of paper on a globe of the world such that the ends of the piece

of paper intersect the path startpoint and endpoint (the paper  should

be  bent  so  that  one  of  its edges lays flat on the surface of the

globe).  The path that this paper makes on the globe  will  be  curved

and  represents  the  great-circle path between Florida and Great Bri-

tain.  By examining the great circle path, we are able to see that the

most  northerly geographical position on the path is at Great Britain.

Since Great Britain is a middle-latitude region, we consult the middle

July 14, 1991

- 13 -

latitude  prediction chart.  If you transmitted to Great Britain at 11

am local time on Thursday, the time at the northpoint (which is  Great

Britain)  would  be 3 or 4 pm (depending on the season).  Since 3 or 4

pm translates to 15:00 or 16:00 in 24-hour  clock  format,  you  would

examine the middle plot line of the chart for Thursday. Note, however,

that the northpoint local time of 15:00 or 16:00 falls  close  to  the

boundary  between  the  middle  plot  line  and the last plot line for

Thursday.  Since this is the case, a more accurate  representation  of

conditions  may  be  obtained  by considering a mix of the middle plot

line with the last plot line (possibly averaging the two plot lines).

An important consideration to note when attempting to  use  these

charts  is the time of sunrise and sunset between the startpoint, end-

point and northpoint.  Since dramatic changes in  ionospheric  charac-

teristics  occur  during these periods, any transmission which crosses

the sunrise or sunset boundaries on  its  way  to  the  endpoint  will

experience  correspondinly  dramatic  changes in behavior and quality.

These charts do not (and could not) account for  these  variations  in

signal quality.  The sunrise and sunset ionospheric anomalies are con-

sidered diurnal components in this discussion.

In many cases, the great-circle path of  the  signal  may  travel

over  more  northerly latitudes than the startpoint and endpoint.  For

example, a transmission between central Canada and Great  Britain  may

result  in  a  great-circle path that passes through the high-latitude

regions before reaching Great Britain, even though both the startpoint

and  endpoint  are middle-latitude stations.  In these cases, the most

northerly position of the great-circle path should be used.

If the signal path (or the two path endpoints) are near the boun-

daries  of two latitudinal zones, a mix of the propagation predictions

for the two latitudinal zones may be required to yield a more accurate

representation of propagation conditions.  For example, if a transmis-

sion were conducted between  Denver  and  Atlantic  City,  which  both

border  as  low  and middle latitude locations, both of the charts for

the low and middle latitude zones should  be  analyzed  and  mixed  in

order  to  determine  the conditions which might be expected over that

path.  Since the distances in this latter example are relatively small

(compared  to  the latter examples), the northpoint of the signal path

will not significantly affect propagation conditions.  This is why  we

only  examined  the  latitude  of  the  startpoint  and endpoint.  For

greater distances, the northpoint must be considered.

2.9.  VHF Propagation Prediction Charts

The prediction of potential VHF DX is not as simple as it is  for

HF.  VHF signals have properties which are not usually affected by the

ionospheric layers.  In our context, "VHF" will  be  considered  those

frequencies  ranging  from  about  50 MHz to 300 MHz.  For information

regarding the major types of VHF propagation which are possible,  con-

sult part I of this document.

A great deal of information can be extracted from the VHF predic-

tion  charts.   Information  pertaining  to  HF communications is also

July 14, 1991

- 14 -

imbedded in these charts.  As was done for the HF  prediction  charts,

the  VHF  predictions are separated into three charts; one for each of

the major latitude zones.

The upper part of the chart forecasts the  quality  of  potential

distant  VHF  signals.   It  does  not  depict  the quality of locally

transmitted VHF signals.  This is  an  important  point  to  remember.

Locally  transmitted  "line  of sight" signals can not be affected and

are not affected by geomagnetic activity, auroral activity,  or  SIDs.

Therefore,  only  the  distant  signals which can be affected by these

phenomena are considered in these charts.

As was the case with the other propagation prediction charts, the

VHF  prediction charts are separated into groups of three 8-hour daily

intervals.  The reference of time used in these charts is LOCAL  time,

not  UT  time.   That  is,  the first line of each day in these charts

represents the period between local midnight and 8 am local time.

To use these charts, simply determine what time it is locally and

examine  the  appropriate  day  in the charts and the appropriate plot

line within that day.  The top portion of the prediction charts define

the  quality  of  VHF  signals  which can be expected over larger dis-

tances.  The bottom chart describes the  probability  of  experiencing

conditions  capable  of  supporting  VHF DX.  Both charts use the same

reference of time (local time).

The  SID  ENHANCEMENT  chart  at  the  upper  right-hand   corner

describes  the  probability  of a SID (sudden ionospheric disturbance)

temporarily enhancing VHF  communications.   The  probability  of  SID

enhancements   increases  with  solar  activity,  but  decreases  with

increasing latitude.  This data is also  of  value  to  the  HF  radio

operator,  since  SIDs  almost  always produce short-wave fades (SWFs)

which can disrupt HF communications.  Since SIDs are sporadic and very

unpredictable  (due to the unpredictable nature of solar flares), they

are  predicted  as  percentages  in  these  charts.   SID-related  VHF

enhancements  do  not occur on the dark-side of the earth.  Neither do

SWFs.  Therefore, these SID prediction  charts  only  apply  to  those

locations which are still well illuminated by the sun.

The AURORAL BACKSCATTER prediction charts are of value to the VHF

radio operator.  Auroral backscattering (as was described in part I of

this  document)  is  possible  at  VHF  frequencies  during  times  of

increased  geomagnetic  and auroral activity.  These prediction charts

define the approximate probabilities of VHF  propagation  via  auroral

backscatter  over  the  various latitudes.  VHF signals which are pro-

pagated via aurorae can travel fairly  large  distances.   Propagation

via  aurorae  is  therefore considered a potential method of DX on VHF

frequencies.  It is, however, a fairly local  and  sporadic  phenomena

and  is  usually  not  a  widely-encountered form of propagation until

auroral and geomagnetic activity reaches significant storm levels.

To make use of these VHF charts, simply consult  the  appropriate

plot  lines  according  to  what day it is and the local time.  The HF

operator may be able to determine what days will prove  less  reliable

July 14, 1991

- 15 -

as  far  as day-time propagation goes by examining the probability for

SID related SWFs.  Most SWFs, however, are only temporary and  do  not

pose  a  significant  threat  to  most HF operators, unless the flares

which produce them are particularly intense.

2.10.  Auroral Activity Predictions

Auroral activity is predicted for the three major latitude  zones

identified  earlier.  Since the auroral oval itself is situated within

the high-latitude zone, the high latitudes will  naturally  experience

significantly more auroral activity than the middle and low latitudes.

The prediction charts for auroral activity  are  most  useful  to

those  people  interested  in  either observing auroral activity, pro-

pagating radio signals using aurorae,  or  for  people  interested  in

determining  the  extent of magnetic fluctuations occurring near areas

of auroral activity.

Since auroral activity migrates  equatorward  during  geomagnetic

storms,  locations  which  may  usually be outside of the auroral zone

itself may occasionally find themselves inside the auroral zone during

periods  of  increased  geomagnetic  activity.   Likewise,  as auroral

activity shifts equatorward, low latitudes may be able to begin  spot-

ting the activity.

These prediction charts can be used to determine whether  or  not

potential  auroral  activity  may  be intense enough to be seen at low

latitudes, or whether auroral activity will be dull  and  inactive  or

bright and very active.

To use the charts, simply examine the appropriate chart  (ex.  if

you're  a middle latitude location, examine the middle-latitude chart)

and select the column on the day you are most interested.   The  first

plot line of each day represents the evening twilight period.  This is

the period between when the sun sets and  before  the  sky  gets  com-

pletely  dark.   The  second  plot line represents the midnight sector

where the sky remains completely  dark  (excluding  effects  of  lunar

phase).   The  last  plot  line  is  the  morning  twilight period and

represents the time when the sky just barely begins to brighten  until

the sun rises.

The phase of the moon is not taken into  consideration  in  these

charts.   The  moon  can  have  a profound effect on the visibility of

auroral activity, but does not affect auroral activity  itself.   That

is,  just  because  the  moon  may  be  blocking  out light of auroral

activity does not mean that  auroral  activity  is  not  in  progress.

Indeed,  intense auroral storms can occur during a full moon as easily

as they can during new moons.  Therefore, these charts  represent  the

occurrence of auroral activity regardless of lunar phase.

The intensity  of  auroral  activity  is  measured  according  to

several  parameters.   Each  of  these parameters are discussed in the

Glossary of Solar Terrestrial Terms.  In their most  basic  form,  the

parameters   (low,   moderate,  high,  etc.)  may  be  considered  the

July 14, 1991

- 16 -

brightness of auroral activity during dark-sky conditions (ie. periods

of  new  moon).   However,  actual visual movements, color changes and

aerial extent are also considered when classifying auroral activity in

these prediction charts.

3.  Monthly Solar Terrestrial Review

Every month, statistics and information regarding solar and  ter-

restrial  activity  for  the preceding month are gathered and compiled

into a document called the Monthly  Solar  Terrestrial  Review.   This

document  contains  not  only  information  regarding  the  nature  of

activity of solar  and  geophysical  phenomena  during  the  preceding

month, but also includes a six month solar cycle outlook.  This can be

of great value to the radio operator who is interested in  determining

what  conditions  might be like six months down the road.  It can also

be of interest to the astronomer who may enjoy searching  for  auroral

activity or solar flares.

In addition to the written summary, a statistical summary of  the

previous  month is given in tabular form, summarizing all of the major

solar parameters (ie.  solar  flux,  sunspot  numbers,  active  region

sizes, numbers and types of flares, etc.).

This report is intended to serve as a general  summary  regarding

activity  and phenomena encountered during the previous month.  It can

provide some interesting results if data from the report is charted or

graphed or statistically analyzed.

4.  Geomagnetic Storm Alert

This alert is posted over the nets whenever magnetic storm condi-

tions  reach or exceed minor storm levels over middle latitudes.  This

alert may be preceded by a warning if a magnetic storm is expected  to

occur but hasn't yet begun.

These alerts always summarize the current level of  activity  and

may  also  include  descriptions  of  outstanding geomagnetic activity

occurring prior to the time the alert was issued.

A brief textual forecast of the expected geomagnetic activity  is

also  included  with  these  reports.   This  effectively serves as an

intermediate forecast which can be of value during  geomagnetic  storm

periods when conditions change rapidly.

Full HF and VHF summaries are included with the storm alerts  and

all following storm information updates.  This information is of value

to those who rely on  ionospheric  and/or  auroral-related  communica-

tions.

July 14, 1991

- 17 -

5.  Availability of Additional Services

Solar Terrestrial Dispatch also supplies other services not  men-

tioned  in this document, which may be of interest or value to certain

individuals or organizations.  One of the additional services provided

are GIC forecast and warning services for individuals or organizations

requiring predictions of  possible  Geomagnetically  Induced  Currents

caused  by  magnetic  storming  over the high and/or middle latitudes.

Forecasts are produced on a weekly  basis,  and  warnings  are  issued

whenever conditions (or expected conditions) warrant.

For more information on this or other services not  mentioned  in

this  document,  feel  free  to  contact Solar Terrestrial Dispatch by

writing to: Solar Terrestrial Dispatch,  Box  357,  Stirling,  Alberta

Canada,  T0K  2E0.  Alternatively, for those of you with access to one

of the large electronic networks, you  may  contact:  oler@hg.uleth.ca

for more information (this is an Internet address).

Significant enhanced services will soon be available  from  Solar

Terrestrial  Dispatch  for  individuals  and researchers interested in

obtaining up-to-the-minute solar terrestrial  data  (ex.  x-ray  data,

proton  data,  geomagnetic data, flare-related data, ionospheric data,

etc).  For more information or questions regarding the availability of

these  or  other services, consult Solar Terrestrial Dispatch as given

above.

A recent addition to the services provided by the  Solar  Terres-

trial  Dispatch  is the Solar Terrestrial Dispatch computer BBS.  This

is a public-access BBS system which regularly obtains fresh  forecasts

and  data  sets.   A  great  deal  of  additional data is available to

registered users of this BBS, including ionospheric data (total  elec-

tron  content  data, MUF forecasts, short-wave fade forecasts and much

more), geomagnetic  data  (real-time  geomagnetic  data  less  than  2

minutes  old,  indices  and  forecasts, etc) and access to other solar

forecasts and data sets (such as real-time x-ray plots, proton  plots,

polar  cap  absorption  plots,  etc).   These  are  all  available  to

registered users of the BBS.  Registration information may be found by

calling  the BBS.  The phone number is: (403) 756-3008.  Baud rates of

300, 1200 or 2400 baud are accepted.  The communications protocol is 8

bit  words,  1 stop bit and no parity.  Unregistered users are able to

access all of the information which is available  over  the  Internet,

Bitnet and Usenet networks.

6.  Concluding Remarks

The solar terrestrial reports which are posted over the  networks

contain a great deal of information.  Understanding them may take some

time.  Applying the  information  contained  in  them  may  take  even

longer.   This  document (part I and II) was developed to help explain

the nature and format of these reports.  It was also developed to help

those  who are interested in interpreting and applying the information

contained in the reports.

July 14, 1991

- 18 -

It is hoped that this document will help those who are interested

in  better  understanding  the  solar  terrestrial reports.  Questions

and/or comments are welcome.  If any further explanations are required

which  have not been adequately covered in this document, feel free to

send an inquiry to  "oler@hg.uleth.ca"  or  leave  a  message  to  the

"Sysop" on our BBS.

We have learned a great deal over the years regarding the impacts

of  solar  activity  on  our terrestrial sphere.  But there is still a

great deal more we need to learn before we can expect  to  master  the

art  of  predicting  the  impacts of solar activity on the earth.  Our

curiosity drives us further and our lust for  knowledge  quickens  our

pace of learning.  With developements of new devices and technologies,

we are steadily edging closer to understanding  both  our  terrestrial

environment and the vast environment of space.  The educational insti-

tutions and research organizations are the backbones of our  knowledge

and  growth.   We  must  therefore respect these institutions, support

them, and encourage them so that our body of knowledge is able to con-

tinue expanding into the limitless realm of science.

July 14, 1991

- 19 -

Table of Contents

Introduction ....................................................    1

The Solar Terrestrial Forecast and Review .......................    2

Summary of Solar Terrestrial Activity ...........................    2

Short Term Solar Terrestrial Forecast ...........................    5

Solar Region Summary ............................................    6

Geomagnetic Activity Summary ....................................    8

10-Day Geomagnetic Activity Forecast ............................    9

Graphical Analysis of Solar Activity ............................    9

20-Day Solar Activity Forecast ..................................   10

HF Radio Signal Propagation Predictions .........................   10

VHF Propagation Prediction Charts ...............................   13

Auroral Activity Predictions ....................................   15

Monthly Solar Terrestrial Review ................................   16

Geomagnetic Storm Alert .........................................   16

Availability of Additional Services .............................   17

Concluding Remarks ..............................................   17

July 14, 1991